# EasyMesh Bring-Up Guide on Raspberry Pi
(RDK OneWifi + Unified-WiFi-Mesh)

This document describes the complete step-by-step procedure followed to successfully build, configure, and run the RDK OneWifi and Unified EasyMesh stack on Raspberry Pi, resulting in successful SSID broadcast and client connectivity.

## 1. Repository Setup

Create project directory and clone repositories:

mkdir easymesh_project
cd easymesh_project
git clone https://github.com/rdkcentral/OneWifi.git
git clone https://github.com/rdkcentral/unified-wifi-mesh.git

## 2. Build Instructions

### 2.1 Setup OneWifi Dependencies (One-time)

cd OneWifi
make -f build/linux/rpi/makefile setup

### 2.2 Build OneWifi

make -f build/linux/rpi/makefile all
Binary: OneWifi/install/bin/OneWifi

### 2.3 Build EasyMesh Components

Controller:

cd unified-wifi-mesh/build/ctrl
make clean
make all

CLI:

cd ../cli
make clean
make all

Agent:

cd ../agent
make clean
make all

## 3. Execution Prerequisites

Install bridge utilities:

sudo apt-get install bridge-utils

Stop wpa_supplicant:

sudo systemctl stop wpa_supplicant
sudo systemctl disable wpa_supplicant
sudo systemctl mask wpa_supplicant

## 4. Network Configuration

Create and configure brlan0 bridge:

sudo brctl addbr brlan0
sudo ip link set dev brlan0 up
sudo ip addr add 192.168.100.1/24 broadcast 192.168.100.255 dev brlan0

Enable IP forwarding and NAT:

sudo sh -c "echo 1 > /proc/sys/net/ipv4/ip_forward"
sudo iptables -t nat -A POSTROUTING -o eth0 -j MASQUERADE
sudo iptables -A FORWARD -i eth0 -o brlan0 -m state --state RELATED,ESTABLISHED -j ACCEPT
sudo iptables -A FORWARD -i eth0 -o brlan0 -j ACCEPT

Start DHCP server:

sudo dnsmasq -i brlan0 -d --dhcp-range=192.168.100.2,192.168.100.200 &

## 5. Database Setup (MariaDB)

Secure MariaDB:

sudo mariadb-secure-installation

Create database:

sudo mysql -u root -p
CREATE DATABASE OneWifiMesh;
USE OneWifiMesh;

## 6. Correct Execution Order

Step 1 – Run OneWifi Core:

cd OneWifi/install/bin
sudo ./OneWifi

Step 2 – Run EasyMesh Controller:

cd unified-wifi-mesh/install/bin
sudo ./onewifi_em_ctrl root@root

Step 3 – Run EasyMesh Agent:

sudo ./onewifi_em_agent

Step 4 – Run CLI:

export LD_LIBRARY_PATH=../lib:$LD_LIBRARY_PATH
sudo -E ./onewifi_em_cli rpi

## 7. Collocated Agent Configuration

In CLI:
1. Navigate to 'Wifi Reset'
2. Press Update
3. Set CollocatedAgentID to wlan0 MAC address
4. Press Apply

## 8. Verification

Check AP mode:
iw dev

Check connected stations:
iw dev wlan0 station dump

SSID: RPI_RDKB-AP0
Password: 12345678
Client IP Range: 192.168.100.x

## Final Result

• OneWifi core running successfully
• EasyMesh Controller running
• EasyMesh Agent registered
• SSID broadcast successful
• Mobile device connected successfully
• DHCP functioning correctly
• Database tables populated